SECTION 13 34 16 - BLEACHERS

GENERAL

ADMINISTRATIVE REQUIREMENTS

Pre-Installation Conference:

Attendance: [Architect,] [Owner,] [Contractor,] [Construction Manager,] [Design/Builder,] installer, and related trades.

Review: Project conditions, manufacturer requirements, delivery and storage, staging and sequencing, and protection of completed work.

SUBMITTALS

Action Submittals:

Shop Drawings: Illustrate products, installation, and relationship to adjacent construction.

Product Data: Manufacturer’s descriptive data and product attributes.

Samples: [Selection samples.] [Verification samples.]

Informational Submittals:

Certificate of Compliance: Certification that installed products meet specified design and performance requirements.

Closeout Submittals:

Maintenance Data.

QUALITY ASSURANCE

Installer Qualifications:

Firm specializing in work of this Section, with minimum [2] [__] years’ experience.

Approved by bleacher manufacturer.

Conform to [applicable accessibility code] [____] for access ramps and wheelchair spaces.

SYSTEM DESCRIPTION

Bleachers:

Size: [15] [21] [27] feet long x [3] [4] [5] rows.

Vertical rise: Maximum 6 inches per row.

Horizontal depth: 21-3/4 inches per row.

Seat height above tread: 16-3/4 inches.

Aisle width: Minimum 48 inches.

Design bleacher system to withstand following loads without damage or deformation of components:

Live load: [100] [__] PSF on gross horizontal area; [120] [__] pounds per linear foot of seat and tread plank.

Perpendicular sway load: [100] [__] pounds per linear foot of seat plank.

Wind load: [30] [__] PSF on vertical projection.

Guardrails: [100] [__] pounds per linear foot vertical; [50] [__] pounds per linear foot horizontal.

Design system under direction of Professional Structural Engineer licensed in State in which Project is located, with minimum [2] [__] years [documented] experience in work of this Section.

WARRANTY

Manufacturer’s [3] [__] year warranty against defects in materials and workmanship.

PRODUCTS

MANUFACTURERS

    - Contract Documents are based on products by PW Athletic Manufacturing Co., Phone 866-402-2893; www.pwathletic.com.

    - Substitutions: [Refer to Division 01.] [Not permitted.]

MATERIALS

Steel Shapes: ASTM A36/A36M.

Aluminum:

Extrusions: ASTM B221, 6061-T6 alloy and temper.

Pipe: ASTM B241, extruded, anodizing quality, 6063 aluminum pipe, Schedule 40.

Wood Mud Sills: Preservative treated for ground contact use.

FABRICATION

Shop assemble in largest practical units.

Welding:

Steel: AWS D1.1/D1.1M.

Aluminum: AWS D1.2/D1.2M.

Framework: Prefabricated [steel] [aluminum] angle type, with crossbracing.

Risers: Extruded aluminum.

Seats: Extruded aluminum, 1-1/2 inches high x 9-1/4 inches deep.

Treads and Steps: Extruded aluminum planks with end caps.

Guardrails:

Railings: 1.66 inch OD aluminum pipe with prefabricated fittings.

Infill: [Galvanized steel chain link mesh.] [Vertical aluminum bars.]

FINISHES

Steel Frame: [Galvanized] [Manufacturer’s standard powder coat, sprayed and basked, [_____] color] ASTM A123/A123M, to 1.25 ounces per square foot.

- ***OR***

- Aluminum Frame: [Mill Finish], [Manufacturer’s standard powder coat, sprayed and baked, [____] color.]

Aluminum Planks: [Anodized coating, AAMA 607.1, to 0.0007 inch minimum thickness, [clear] [dark bronze.] [black.]] [Manufacturer’s standard powder coat, sprayed and baked, [____] color.]

EXECUTION

INSTALLATION

Install in accordance with manufacturer's instructions and approved Shop Drawings.

Installation Tolerances:

Variation from location: Plus or minus 1/2 inch.

Variation from level or true plane: Maximum 1/4 inch in 10 feet.

- END OF SECTION