Pre-Engineered Fabric Shade Products

Part 1 – General

1.1 	Related Documents

Drawings and general provisions of the Contract, including General Conditions and Division 1 	Specifications Sections, apply to this section.

1.2 	Summary

The shade structure contractor shall be responsible for design, engineering, fabrication and supply of 	the work specified herein.  The intent of this specification is to have only one manufacturer be 	responsible for the aforementioned functions.

1.3 	Submittals

1.3.1	 Pre-Bid Submittals

A. 	Provide proof of installed reference sites with structures for similar scope of project and 			installation that are engineered to International Building Code (IBC) specifications.  Include in 		reference list of structure dimensions with install dates and project locations.

B. 	Provide information to establish desired fabric color and power coat color.

C. 	Provide proof of all quality assurance items including:

1.	A list of at least six (6) public municipal installations where manufacturer’s 				product as proposed pursuant to this bid has been installed and has been in 				continuous use 	for a minimum of five (5) years each.

2.	All manufacturers shall provide proof of a minimum $ 2,000,000.00 (AG) General 			Public Liability Insurance, $ 2,000,000.00 Professional Liability (PL) insurance, 				$ 100,000.00 Inland Marine Insurance, and additional $ 5,000,000.00 Umbrella Liability 			insurance.

1.3.2 	Award of Contract Submittals

A. 	Make available wet-sealed structural engineering drawings and calculations

B. 	Provide fabric color and powder coat color selections for final order.

1.4 	Project Conditions

A. 	Field Measurements: verify layout information for shade structures shown on the drawings in 		relation to the property survey and existing structures.  Verify locations by field 				measurements prior to construction.

1.5 	Warranty

A. 	The successful bidder shall provide a one (1) year warranty on all labor and materials.

B. 	A supplemental non-prorated warranty from the manufacturer shall be provided for a period 		of ten (10) years on fabric including stitching and twenty (20) years on the structural integrity 		of the steel, from date of substantial completion.

C. 	The warranty shall not deprive the Owner of other rights the Owner may have under the 		provisions of the Contract Documents, and will be in addition to, and run concurrent with, 		other warranties made by the Contractor under  requirements of the Contractor documents.

D. 	Because of surety requirements, any performance and payment bond that might be required 		will cover only the first year of the warranty. The manufacturer’s warranty will be a separate 		document and will be executed at the time of completion of the work.

Part 2 – Products

2.1 	General

The shade products shall be designed and manufactured to the most exacting specifications by skilled 	craftsmen, and certified by Professional Engineers for structural soundness of designs.  All shade 	products are shipped knocked-down, with complete assembly instructions, and ready for easy in-field 	installation.

Bidder’s products must be completely manufactured entirely in its own factory by its own employees, 	including powder-coating, thereby ensuring complete quality control.  Bidder must certify that no 	aspect of its production – including powder-coating – is contracted out to third parties.

The proposed structure(s) manufactured by Shade Systems, Inc.TM or approved equal, shall be modular and pre‐fabricated, and include the structural steel frame, fabric roof, steel cables and all fasteners. 	Manufactured by: Shade Systems, Inc.

4150 SW 19th Street

Ocala, FL 34474

(800) 609-6066 Tel

(352) 237-2256 Fax

Contact:  Jeremy Purkis

Email:  Jeremy@shadesystemsinc.com

www.shadesystemsinc.com

B. 	Or Equal: Standard for approved equal.  Ten (10) day prior approval required for substitution 		of product design, materials and features specified above.  Submittals must include plans, 		drawings, cut 	sheets, material data sheets, testing results and samples.  Bids failing to meet 		this requirement will be deemed non-responsive.

C. 	Structures are engineered to meet or exceed the requirements of International Building Code 		(IBC), and the following standard specifications:

Wind Speed (Frame only):  165 M.P.H.

Wind Speed (Frame w/canopy):  90 M.P.H.

Live Load:    	None

Snow Load:   	None

Optional designs with greater wind speeds, live loads, and snow loads are available.

D. 	Material: All materials shall be structurally sound and appropriate for safe use.  Product 			durability shall be ensured by the use of corrosion-resistant metals such as stainless steel, and 		coatings such as zinc-plating, galvanizing, and powder-coating on steel parts, subject to the 		Product-Specific requirements.  Fabrics used shall include UV-stabilizers and fire retardants 		for longevity and safety.

E. 	Packaging: All metal posts, rafters and beams shall be wrapped in plastic and cardboard to 		protect the powder coat finish during shipping.

F. 	Weldments: All tubing members are factory-welded by Certified Welders to American 			Welding Society (AWS) specifications and to the highest standards of quality workmanship.  		Weldments are finished with a zinc-rich galvanized coating. No field welding is required in the 		assembly of the shade products.

G. 	Posts, Structural Frame Tubing, and Hardware: All tubing used shall be cold-formed and 			milled per ASTM A-135 and ASTM A-500.  Material testing is in accordance with ASTM E-8.  		Minimum yield is 40,000 psi with a minimum tensile strength of 45,000 psi on all posts.  			All tubing shall be pre-cut to appropriate lengths, and all outside surfaces shall be 			galvanized, with an interior corrosion-resistant zinc-rich coating. Where required, support 		pipes shall be schedule 40 hot-dip galvanized or 	powder-coated black steel.  All fastening 		hardware shall be stainless steel.

H. 	Architectural Powder-coating Process:  All powder-coated parts undergo a rigorous multi-		step process to ensure colorfastness and durability per the specific sequential steps itemized 		below.  All parts are completely sandblasted, pre-treated, and coated with coastal primer 		prior to powder coating. Powder-coating is then electrostatically applied and oven-cured at 		375 to 425 degrees Fahrenheit.  Powders shall meet or exceed ASTM standards for Adhesion, 		Hardness, Impact, Flexibility, Overbake Resistance, and Salt Spray Resistance.  Colors shall be 		specified.

The following seven (7) specific steps shall occur in sequence:

- Sandblasting.  All powder-coated parts shall be completely sandblasted with the use of 80 grit garnet abrasives.

- Mechanical smoothing.  A traditional mechanical method shall be used for removing remaining foreign matter for surface preparation by use of sanding, grinding, and rounding rough edges to smoothness.

- Initial Surface Preparation.  A heavy-duty liquid cleaner such as Calvary Industries Inc Cal Clean 675 shall be applied for initial surface preparation.

- Corrosion resistant Coating. A liquid detergent iron phosphate, such as Calvary Industries Inc, Cal Prep 63, shall be applied, thereby resulting in a superior quality corrosion resistant coating.

- Final Surface Preparation. All parts shall then be sealed using a reactive, non-chrome sealer product such as Calvary Industries, Advantech S1488E Sealer.  The sealer enhances corrosion protection and increases paint adhesion, effectively increasing salt spray hours on all metal substrates.

- Coastal Primer.  Prior to powder-coating, a rust inhibiting coastal primer shall be applied on all parts, such as PPG EnvirocronTM.  The coastal primer coating provides a combination of good physical and chemical resistance properties, and is the ideal solution for smooth, low-bake durability and physical property requirements for the most demanding environments.

Primer attributes:

Gloss (ASTM D-523):			0-10 @ 60o

Adhesion (ASTM D-3359):		100% (5B Pass)

Hardness (ASTM D-3363):		2H Pencil (Eagle)

Impact Resistance (ASTM D-2794):	80 In.-lbs. Direct

Conical Mandrel (ASTM D-522):		1⁄8" - No Cracking

Salt Spray (ASTM B-117):		4000 Hours Pass

1000 Hours (degrease only)

Humidity (ASTM D-1735):		100F, 100% RH-2000+ Hours

Scab Corrosion (SAE-J2334):		120 Cycles - Pass

Film Properties (Thickness):		2 mils

- Application of Powder-Coating. Lastly, PPG EnvirocronTM Ultradurable powder coatings shall be used to provide a combination of excellent physical and chemical resistance properties, outstanding resistance to outdoor weathering, and a durable and uniform final coat.

Powder Coat Characteristics:

Gloss (ASTM D-523):			80 Minimum @ 20o

Gloss (ASTM D-523):			80 Minimum @ 60o

Adhesion (ASTM D-3359):		100% (5B Pass)

Hardness (ASTM D-3363):		2H Pencil (Eagle)

Impact Resistance (ASTM D-2794):	40 In.-lbs. Direct

20 In.-lbs. Reverse

Conical Mandrel (ASTM D-522):		1⁄8" Mandrel - No Cracking

Salt Spray (ASTM B-117):		1000 Hours Pass

< 1⁄8" Scribe Creep

No Blisters

Humidity (ASTM D-1735):		1000 Hours Pass

< 1⁄16" Scribe Creep

No Blisters

Film Properties (Thickness):		3 mils

I. 	Standard Footings: Footings shall be designed per stringent International Building Code (IBC) 		for the specified structure.  Columns will be provided as standard direct embedment.  Other 		footing 	designs are available.

J. 	Roofing: Structural frames and/or fabric sails are designed by Shade Systems only for use with CoolNetTM polyethylene shade fabric. Fabric is attached to frame or columns using stainless steel and clear vinyl coated cable. Cable fasteners are zinc-plated copper for maximum corrosion resistance.

2.2 	Fastening System (Frame Structure)

A. 	CoolnetTM Shade Fabric shall be delivered complete with independent cables pre-inserted 		in fabric hems. Each cable shall be looped and clamped at each end. Fastening System to 		consist of the Turn-N-SlideTM fastening device which is factory installed at each roof rafter 		corner.  The Turn-N-Slide features a concealed mechanism which allows the attachment 	hook 		and sleeve at each rafter corner to move along a track in the rafter. Cables are attached to 		hook which is welded to the moving sleeve, thereby distributing tension evenly over rafters 		and not directly onto the mechanism. Rafters are sealed with no penetrations on the top side, 		thereby preventing water from entering.  Such moving sleeve with hook allows the looped 		ends of each cable to slide over the hook when the sleeve is at its upper position, and 			then by turning the concealed fastener 	within the rafter, moves the sleeve with hook 			outward (toward end of rafter), thereby tensioning the cables and securing the fabric at the 		proper tautness. A locking cap is secured at the end of each rafter with a vandal-resistant bolt 		(special wrench provided by the manufacturer) to prevent unauthorized access to the Turn-N-		Slide mechanism. To remove the canopy, the cap is removed, and the mechanism rotated 		counter- clockwise. The sleeve with hook moves inward (toward peak of roof), 				thereby de-tensioning the cables, and allows fast removal of the canopy.  Continuous one-		piece cables, cables which are not independent per side and pre-looped and clamped at the 		factory, and/or cables which must be tensioned with the use of turnbuckles or tools not 			provided by the manufacturer are not acceptable. Structures which do not feature the 			Fastening Mechanism on each and every rafter, or fastening mechanisms which do not 			feature a sealed top rafter and moving outer sleeve such as the Turn-N-Slide, are not 			acceptable.

B. 	Fastening System Instructional Video: Product must be delivered complete with a minimum 5-		minute 	instructional video on an USB Flash Drive.  Video must show the viewer the exact 		procedure for removing and re-attaching canopy using an actual shade structure in the 			field.  Submittals which do not include the video on an USB Flash Drive are not acceptable.

2.3 	Fastening System (Sail Structure)

A.	CoolNetTM Shade Fabric shall be delivered complete with fastening system installed. Fastening 		System to consist of factory-formed stainless steel tensioning plates pre-attached to fabric 		canopies at each corner, and cables per the above hemmed into the fabric at the factory and 		terminating in the bracket. Posts shall be equipped with an adjustable 360-degree swivel and 		pivot attachment mechanism to which the tensioning plate fastens. Tensioning plate includes 		a stainless steel adjustment bolt which, when turned, tensions the fabric for a taut fit. Fabrics, 		cables, and brackets which are not pre-assembled at the factory are not acceptable.  Cables 		which attach to posts with u-bolts or ‘S’ hooks, and which do not use a stainless steel 			bracketing system similar to the above are not acceptable.

2.3 	Fabric

A. 	Shade Fabric: Knitted of monofilament and tape construction high density polyethylene with 		Ultra-Violent (U.V.) stabilizers and flame retardant. CoolNetTM offers the ultimate 			combination of maximum sun protection, strength and durability to ensure maintenance free 		long-life performance.  UV- Block Factor varies by standard color offered from 90% to 97%.

CoolnetTM Properties:

Nominal Fabric Mass:				Min. 340 g/m2  //  10 oz/yd2

Fabric Thickness: 	     ASTM D5199-12	.06 inch

Temperature Range: 				-50OF to 248OF

Roll Width:					9 ft. 10 in.

Roll Length: 					131 ft.

Tensile Strength:	     ASTM D5034-09	Warp (189.1 lbf) / Weft (462.3 lbf)

Elongation:		     ASTM D5034-09	Warp (88.7%) / Weft (49%)

Tongue Tear:		     ASTM D2261-13	Warp (39.6 lbf) / Weft (43 lbf)

Burst Strength:		     ASTM D6797-15	408.0 lbf

Flammability:		     ASTM E-84 Class A

Lead:						PASS

Phthalate:    					PASS

CoolNetTM Shade Fabrics meet the most stringent Fire Standards for shade fabrics including 		CSFM 1237.1 and NFPA 701 across all color variants.

All hems and seams are double row lock stitched using exterior grade UV- stabilized 			polyethylene GORE™ TENARA™ sewing thread (GORE and TENARA are trademarks of W.L. 		Gore & Associates).

| Color | Weight (oz/yd²) | Shade Factor % | UVR Block % |
| --- | --- | --- | --- |
| Canary Yellow | 10 | 78 | 93 |
| Oyster Grey | 10 | 80 | 92 |
| Eggshell White | 10 | 78 | 94 |
| Lime Green | 10 | 88 | 91 |
| Fire Orange | 10 | 81 | 92 |
| Blizzard Blue | 10 | 87 | 93 |
| Grape Purple | 10 | 88 | 91 |
| Desert Sand | 10 | 86 | 93 |
| Rivergum Green | 10 | 90 | 94 |
| Bright Red | 10 | 80 | 90 |
| Mocha Brown | 10 | 94 | 96 |
| Charcoal Grey | 10 | 94 | 94 |
| Bora Bora Blue | 10 | 90 | 93 |
| Lagoon Blue | 10 | 90 | 94 |
| Stormy Grey | 10 | 95 | 97 |
| Barley Brown | 10 | 89 | 94 |
| Brick Red | 10 | 91 | 92 |
| Silver Grey | 10 | 90 | 92 |
| Light Blue | 10 | 91 | 94 |
| Navy Blue | 10 | 93 | 94 |
| Aquatic Blue | 10 | 88 | 92 |
| Forest Green | 10 | 93 | 94 |

Part 3 – Execution

3.1 	Installation

Installations of shade structure(s) shall be performed by an installer who shall comply with the 	manufacturer’s instructions for assembly, installation, and erection, per approved drawings.

A. 	Concrete

1.	Concrete work shall be executed in accordance with the latest edition of the 				American Concrete Building Code, ACI 318.

2. 	All reinforcement shall confirm to ASTM A-615, Grade 60.

3.	Reinforcing steel shall be detailed, fabricated, and placed in accordance with the 			latest ACI Detailing Manual, and Manual of Standard Practice.