Teak Protector

Use to maintain the original color of teak for a longer time

Reasons to use over Teak-Oil/Sealer

- Teak Protector maintains the light-brown color of teak for a longer time:

  - Up to 4 times longer than teak-oil

  - Up to twice as long as teak sealers

- Teak Protector is environment-friendly because it is 100% water-based; no toxic fumes; non-flammable; eco-safe.

- Teak Protector is not an oil and not greasy which allows clothing to stay clean.

- Teak Protector is resistant to mold and prevents black spots on teak.

- Teak Protector is fast and easy to apply.

Instructions for Use

- Before using the Teak Protector, eye protection and rubber gloves should be worn. Cover the ground under your work area before starting.

- Grey teak or teak treated with teak-oil must first be cleaned with Teak Cleaner

- New teak: first weather for 2 weeks outside before treatment.

- If cleaned wood feels rough, sand lightly before treating with Teak Protector. Make sure the teak is 100% dry and free of dust.

- Shake and stir Teak Protector at least a minute before use. Insufficient shaking can lead to uneven weathering.

- on a small area first (e.g. the back of a leg) to judge if the color meets your expectations.

- Apply a very thin layer of TEAK PROTECTOR evenly over the furniture witha dust free cloth. Apply a second very thin coat after 30 minutes drying time.

- It is recommended that you treat your furniture with TEAK PROTECTOR at least once a year.