Teak Shield

Use to shield teak from stains and markings

- Before using the Teak Shield, eye protection and rubber gloved should be worn. Cover the ground under your work area before starting.

- New Teak: First clean without Teak Cleaner to remove some of the natural teak oils (or weather for 2 weeks outside before treatment).

- Before using the Teak Shield on dark gray teak, clean the wood with Teak Cleaner. If cleaned wood feels rough, sand lightly before treating with Teak Shield.

- Make sure the teak is 100% dry and free of dust.

- Shake for at least one minute before use. Apply a thin layer of Teak Shield with a soft cloth.

- Allow to dry for 4 hours and then apply a second thin coat of Teak Shield. Let the teak dry for 48 hours before exposure to moisture.

- It is recommended that you treat your teak furniture with Teak Shield once a year. Tabletops should be treated more often.